# Part 1: Side Camera System – Functional Requirements

## Overview

The Side Camera System is designed to verify whether a mattress or product belongs to the correct category by comparing the outputs of three independent verification algorithms. The system uses two cameras and combines QR code information, OCR-based bill reading, and texture detection to ensure that all sources identify the same product.

## System Architecture

### Camera Configuration

### Camera 1 – QR Code Scanner

- Dedicated camera for QR code scanning.

- Captures the QR code from the product.

Extracts:

  - Item ID

  - Batch Number

  - Product Variety (e.g., MaxiPlus)

  - Any additional encoded metadata available in the QR code.

### Camera 2 – OCR & Texture Detection

This camera performs two tasks simultaneously:

- OCR (Bill/Label Reading)

  - Reads the printed bill, label, or product information.

  - Extracts the product name/variety and other relevant text.

- Texture Detection

  - Captures the product surface.

  - Uses the trained texture classification model to identify the product category or variety.

## Software Modules

The implementation consists of two independent processing modules.

### Module 1 – QR Processing

Responsible for:

- Capturing the QR image.

- Reading and decoding the QR code.

- Extracting product information.

- Returning structured QR data.

### Module 2 – OCR & Texture Processing

Responsible for:

- Capturing an image from Camera 2.

- Running OCR on the captured image.

- Running the trained texture detection model.

- Returning both OCR and texture prediction results.

The trained AI models for OCR and texture detection are already available and will be integrated into this module.

## Product Verification Logic

The system compares the outputs of the following three verification methods:

- 1. QR Code Reading

- 2. OCR Reading

- 3. Texture Detection

### Example (Pass)

| QR Code | OCR | Texture Detection |
| --- | --- | --- |
| MaxiPlus | MaxiPlus | MaxiPlus |

Result: PASS — since all three algorithms identify the same product category, the product is successfully verified.

## Mismatch Detection

If any of the three verification methods returns a different product category, the system must generate a warning. The warning should clearly indicate:

- Which verification method failed.

- Expected product category.

- Detected product category.

### One Mismatch

| QR Code | OCR | Texture | Result |
| --- | --- | --- | --- |
| MaxiPlus | MaxiPlus | UltraPlus | FAIL |

- FAIL

- Texture Detection Mismatch

### Two Mismatches

| QR Code | OCR | Texture | Result |
| --- | --- | --- | --- |
| MaxiPlus | UltraPlus | UltraPlus | FAIL |

- FAIL

- OCR Mismatch

- Texture Detection Mismatch

### Three Mismatches

| QR Code | OCR | Texture | Result |
| --- | --- | --- | --- |
| Deluxe | UltraPlus | MaxiPlus | FAIL |

- FAIL

- QR Code Mismatch

- OCR Mismatch

- Texture Detection Mismatch

## Database Design

All inspection results will be stored in a JSON-based database. Each inspection record should contain the following information:

### Inspection Information

- Inspection ID (Unique)

- Timestamp

- Processing Status (Pass/Fail)

- Overall Verification Result

### QR Code Data

- Item ID

- Batch Number

- Product Variety

- QR Code Value

- Additional QR Metadata (if available)

### OCR Results

- Extracted Product Name

- Extracted Text

- OCR Confidence Score

### Texture Detection Results

- Predicted Product Category

- Prediction Confidence

- Model Version

### Comparison Results

- QR Result

- OCR Result

- Texture Result

- Matching Status

- List of Mismatches

- Remarks

### Image Storage

Store references (paths) to:

- QR Image

- OCR Image

- Texture Image

- Processed Images (if generated)

Instead of storing images directly in the JSON file, the JSON should contain the file paths where the images are stored. Example:

- qr_image_path

- ocr_image_path

- texture_image_path

## Dashboard Requirements

A dedicated dashboard will be developed for the Side Camera System.

### Live Camera View

The dashboard should display:

- Live feed from Camera 1 (QR Scanner)

- Live feed from Camera 2 (OCR & Texture)

### Process Button

When the operator clicks the Process button, the system should:

- 1. Capture images from both cameras.

- 2. Decode the QR code.

- 3. Run OCR.

- 4. Run Texture Detection.

- 5. Compare all three outputs.

- 6. Generate the final verification result.

- 7. Display the result on the dashboard.

- 8. Save the inspection record to the database.

### Result Panel

The dashboard should display:

- PASS / FAIL status

- QR Code Result

- OCR Result

- Texture Result

- Product Variety

- Item ID

- Batch Number

- Timestamp

- Confidence Scores

- List of mismatches (if any)

### Inspection History

The dashboard should maintain a searchable inspection history. Users should be able to search records using:

- Item ID

- Product Variety

- Batch Number

- Date

- Timestamp

- Inspection Status (Pass/Fail)

Selecting a record should display:

- All inspection details

- Captured images

- Verification results

- Mismatch details

- Confidence scores

## Additional Features

The system should also include:

- Automatic timestamp generation for every inspection.

- Unique inspection ID for traceability.

- Image archiving with organized folder structure.

- JSON-based inspection logs.

- Confidence scores for OCR and texture predictions.

- Error handling for unreadable QR codes or OCR failures.

- Camera connection status monitoring.

- Processing progress indicator.

- Manual reprocessing option for failed inspections.

- Export inspection history (JSON/CSV) for reporting.

- Audit logs for all inspection activities.

- Scalable database structure to support future product categories and additional verification algorithms.

## Workflow Summary

- Operator places the product in the inspection area.

- Camera 1 scans the QR code.

- Camera 2 captures images for OCR and texture detection.

- QR processing module extracts product details.

- OCR module reads product information.

- Texture detection module classifies the product.

- The system compares all three outputs.

- If all outputs match, the inspection is marked as PASS.

- If one or more outputs differ, the inspection is marked as FAIL, and the mismatched verification sources are highlighted.

- The inspection data, captured images, comparison results, and timestamp are stored in the JSON database.

- The dashboard updates with the latest inspection result and makes it available for future search and review.

# Part 2: Top Camera System – Functional Requirements

## Overview

The Top Camera System is responsible for inspecting the physical characteristics of the mattress from a top view. Its primary functions are to measure the mattress dimensions and read the corner label. The information extracted from the top camera is then correlated with the results from the Side Camera System (QR Code, OCR/Bill Reading, and Texture Detection) to perform a final product verification.

## System Architecture

### Top Camera

The Top Camera performs two primary inspection tasks:

- 1. Dimension Measurement

  - Capture a top-view image of the mattress.

  - Measure the mattress dimensions (Length, Width, and optionally Height/Thickness if supported).

  - Compare the measured dimensions with the expected dimensions for the identified product.

- 2. Corner Label Detection & Reading

  - Detect the corner label attached to the mattress.

  - Read the label using OCR.

Extract relevant information such as:

  - Product Name / Variety

  - Product Code (if available)

  - Size

  - Manufacturing Details (if available)

  - Any other printed information present on the label.

## Software Modules

### Module 1 – Dimension Measurement

Responsible for:

- Capturing the top-view image.

- Detecting mattress boundaries.

- Measuring the mattress dimensions.

- Returning the measured values.

### Module 2 – Corner Label Reading

Responsible for:

- Detecting the corner label.

- Cropping the label region.

- Running OCR on the label.

- Extracting product information.

- Returning structured label data.

## Database Storage

Every inspection performed by the Top Camera must be stored in the inspection database (JSON). Each inspection record should include:

### Inspection Information

- Inspection ID

- Timestamp

- Processing Status

- Camera Name (Top Camera)

### Dimension Results

- Measured Length

- Measured Width

- Measured Thickness (if applicable)

- Expected Dimensions

- Dimension Verification Status (Pass/Fail)

### Corner Label Results

- Extracted Product Name

- Extracted Product Variety

- Extracted Product Code

- Full OCR Text

- OCR Confidence Score

### Image Storage

Store references (paths) for:

- Original Top Camera Image

- Cropped Corner Label Image

- Processed Image (if generated)

The JSON database should only store image file paths rather than the image data itself. Example fields:

- top_image_path

- corner_label_image_path

- processed_image_path

## Verification Logic

The information extracted from the Top Camera should be correlated with the inspection results from the Side Camera System. The following four product identification sources must match:

- 1. QR Code Reading

- 2. OCR/Bill Reading

- 3. Texture Detection

- 4. Corner Label Reading

### Example (Pass)

| QR Code | Bill OCR | Texture Detection | Corner Label |
| --- | --- | --- | --- |
| MaxiPlus | MaxiPlus | MaxiPlus | MaxiPlus |

Result: PASS — since all four verification methods identify the same product, the inspection is successful.

## Mismatch Detection

If one or more verification methods identify a different product, the system should generate a warning. The warning must clearly indicate:

- Which verification source is mismatched.

- Expected product.

- Detected product.

### One Mismatch

| QR Code | Bill OCR | Texture | Corner Label | Result |
| --- | --- | --- | --- | --- |
| MaxiPlus | MaxiPlus | MaxiPlus | UltraPlus | FAIL |

- FAIL

- Corner Label Mismatch

### Two Mismatches

| QR Code | Bill OCR | Texture | Corner Label | Result |
| --- | --- | --- | --- | --- |
| MaxiPlus | UltraPlus | MaxiPlus | UltraPlus | FAIL |

- FAIL

- Bill OCR Mismatch

- Corner Label Mismatch

### Multiple Mismatches

The system should identify and display every mismatched verification source individually.

## Dashboard Requirements

A dedicated section for the Top Camera should be available within the inspection dashboard.

### Live Camera View

Display the live feed from the Top Camera.

### Process Button

When the operator presses the Process button, the system should:

- 1. Capture the top-view image.

- 2. Measure mattress dimensions.

- 3. Detect the corner label.

- 4. Perform OCR on the corner label.

- 5. Compare the corner label information with:

  - QR Code result

  - Bill OCR result

  - Texture Detection result

- 6. Generate the final verification result.

- 7. Save all inspection data and images to the database.

### Result Panel

Display:

- PASS / FAIL Status

- Measured Dimensions

- Expected Dimensions

- Dimension Verification Result

- Corner Label Information

- OCR Confidence

- Timestamp

- Mismatch Details (if any)

## Inspection History

The dashboard should allow users to search previous inspections using:

- Inspection ID

- Item ID

- Product Variety

- Batch Number

- Date

- Timestamp

- Inspection Status

Selecting an inspection should display:

- Top Camera Image

- Corner Label Image

- Measured Dimensions

- OCR Results

- Verification Results

- Mismatch Details

## Additional Features

The Top Camera System should also include:

- Automatic timestamp generation.

- Unique inspection ID.

- Image archiving.

- JSON-based inspection records.

- Confidence scores for OCR.

- Dimension tolerance checking.

- Error handling for unreadable labels.

- Camera connection status monitoring.

- Processing progress indicator.

- Manual reprocessing option.

- Audit logging for inspection history.

## Workflow Summary

- Operator positions the mattress under the Top Camera.

- The system captures a top-view image.

- Mattress dimensions are measured.

- The corner label is detected and read using OCR.

- The extracted corner label information is compared with QR Code Reading, Bill OCR Reading, and Texture Detection Result.

- The system verifies whether all four identification methods correspond to the same product.

- If all verification results match, the inspection is marked as PASS.

- If one or more verification sources differ, the inspection is marked as FAIL, and the mismatched sources are highlighted.

- The inspection record, extracted information, measured dimensions, captured images, and timestamp are stored in the JSON database for future retrieval and traceability.