MATTRESS INSPECTION SYSTEM

Combined Functional Requirements Specification

Side Camera System  +  Top Camera System

Engineering Handoff Document

Version 1.0

August 3, 2026

Document Control

| Version | Date | Author / Source | Description |
| --- | --- | --- | --- |
| 0.1 | — | Side Camera System spec | Original functional requirements for QR / OCR / Texture verification (3-source model). |
| 0.2 | — | Top Camera System spec | Original functional requirements for dimension measurement and corner label verification (4-source model). |
| 1.0 | Aug 3, 2026 | Consolidated by Claude for handoff | Merged both specifications into a single functional requirements document; added assumptions, open questions, glossary, and a unified data model for engineering handoff. |

Table of Contents

(Right-click the table above and choose “Update Field” in Word to populate page numbers.)

# 1. Introduction

## 1.1 Purpose of This Document

This document consolidates two previously separate functional requirements specifications — the Side Camera System and the Top Camera System — into a single reference for engineering handoff. Both subsystems inspect the same physical product (a mattress) from different vantage points and feed into one shared verification and database layer, so this merged view is intended to be the single source of truth going forward rather than two documents that must be read and reconciled independently.

In addition to combining the two original specifications, this document adds detail that is useful specifically for a handoff: a unified data model, an explicit description of how the 3-source (Side) and 4-source (Top) verification models relate to one another, a consolidated feature list, and a set of assumptions and open questions that the receiving engineering team should confirm before implementation begins.

## 1.2 System Overview

The Mattress Inspection System uses three cameras across two physical subsystems to confirm that a mattress on the inspection line matches its declared product category. Each camera contributes one or more independent identification signals; the system is considered verified only when all signals agree.

| Subsystem | Camera(s) | Identification Signals Produced |
| --- | --- | --- |
| Side Camera System | Camera 1 (QR Scanner) Camera 2 (OCR & Texture) | QR Code data, OCR / bill text reading, Texture classification |
| Top Camera System | Top Camera | Corner label OCR reading, physical dimension measurement |

Together, these produce four independent product-identification sources — QR Code, Bill/Label OCR, Texture Detection, and Corner Label — plus one physical-tolerance check (dimensions) that is verified separately from product identity.

## 1.3 Scope

### In Scope

- Image capture, decoding, OCR, and classification logic for all three cameras.

- Comparison logic that determines PASS/FAIL status across identification sources.

- The JSON-based inspection database and image archival structure.

- Dashboard requirements for live camera views, processing, results, and searchable history.

### Out of Scope

- Camera hardware selection, mounting, and lighting design.

- Training of the OCR and texture-detection AI models (assumed pre-trained and delivered as-is).

- Conveyor / PLC integration and physical line control, unless explicitly added in a future revision.

## 1.4 Intended Audience

Backend/API engineers implementing the processing modules and database; frontend engineers building the dashboard; QA engineers writing test plans; and any new team member picking up this project who was not involved in the original two specifications.

# 2. System Architecture

## 2.1 High-Level Architecture

| Camera | Location | Primary Function | Output(s) |
| --- | --- | --- | --- |
| Camera 1 | Side, dedicated | QR code scanning | Item ID, Batch Number, Product Variety, additional QR metadata |
| Camera 2 | Side, shared | OCR (bill/label) + Texture classification | Extracted product name/text; predicted product category + confidence |
| Top Camera | Top, overhead | Dimension measurement + corner label OCR | Measured L/W/(H); Product Name/Variety/Code from label |

## 2.2 Side Camera System

### Camera 1 — QR Code Scanner

Dedicated camera whose sole function is to capture and decode the QR code affixed to the product. Extracted fields:

- Item ID

- Batch Number

- Product Variety (e.g., MaxiPlus)

- Any additional encoded metadata present in the QR payload

### Camera 2 — OCR & Texture Detection

A single camera feed is used for two independent tasks that run against the same captured image:

- OCR (Bill/Label Reading): reads the printed bill, label, or product information and extracts the product name/variety and other relevant text.

- Texture Detection: captures the product surface and runs the trained texture-classification model to identify product category or variety.

## 2.3 Top Camera System

### Dimension Measurement

Captures a top-view image and measures Length, Width, and optionally Height/Thickness, then compares the measurement against the expected dimensions for the identified product.

### Corner Label Detection & Reading

Detects and reads the corner label attached to the mattress via OCR, extracting:

- Product Name / Variety

- Product Code (if available)

- Size

- Manufacturing details (if available)

- Any other printed information on the label

# 3. Software Modules

The system is implemented as four independent processing modules — two per subsystem. Each module owns one camera capture and returns a structured result to the comparison layer.

| Module | Responsibilities | Input | Output |
| --- | --- | --- | --- |
| 1. QR Processing (Side) | Capture QR image; decode QR code; extract product information | Camera 1 frame | Structured QR data object |
| 2. OCR & Texture Processing (Side) | Capture image from Camera 2; run OCR; run trained texture-detection model | Camera 2 frame | OCR result + texture prediction + confidence |
| 3. Dimension Measurement (Top) | Capture top-view image; detect mattress boundaries; measure dimensions | Top Camera frame | Measured L / W / (H) |
| 4. Corner Label Reading (Top) | Detect corner label; crop label region; run OCR; extract product info | Top Camera frame | Structured label data object |

Note: The trained AI models for OCR and texture detection are already available and are to be integrated into Module 2 as-is; no model training work is in scope for this handoff.

# 4. Product Verification Logic

## 4.1 Two-Stage Verification Model

The two source specifications each defined a comparison model at a different scope: the Side Camera System compares three sources, and the Top Camera System compares four. Combined, the system operates as two stages that share the same underlying comparison logic:

- Stage 1 — Side verification (3 sources): QR Code, OCR/Bill Reading, and Texture Detection are compared as soon as the Side Camera System finishes processing.

- Stage 2 — Full verification (4 sources): once the Top Camera adds the Corner Label reading, all four sources are compared to produce the final, authoritative PASS/FAIL result for the inspection record.

Note: Whether Stage 1 is surfaced to the operator as an interim result or only logged internally pending Stage 2 is not specified in either source document — see Section 10, Open Questions.

## 4.2 Verification Sources

- 1. QR Code Reading (Camera 1)

- 2. OCR / Bill Reading (Camera 2)

- 3. Texture Detection (Camera 2)

- 4. Corner Label Reading (Top Camera)

Dimension measurement is checked separately as a tolerance pass/fail and is not one of the four identity-matching sources (see Section 4.5).

## 4.3 Pass Example

| Source | Detected Value | Status |
| --- | --- | --- |
| QR Code | MaxiPlus | Match |
| Bill OCR | MaxiPlus | Match |
| Texture Detection | MaxiPlus | Match |
| Corner Label | MaxiPlus | Match |

Result:

PASS — all four verification methods identify the same product.

## 4.4 Mismatch Detection

If any verification source returns a product category different from the others, the system generates a warning that clearly states:

- Which verification source(s) are mismatched

- The expected product category (the majority / reference value)

- The detected product category at the mismatched source

### Mismatch Scenarios

| Scenario | QR Code | Bill OCR | Texture | Corner Label | Result |
| --- | --- | --- | --- | --- | --- |
| 1 mismatch | MaxiPlus | MaxiPlus | UltraPlus | MaxiPlus | FAIL |
| 1 mismatch | MaxiPlus | MaxiPlus | MaxiPlus | UltraPlus | FAIL |
| 2 mismatches | MaxiPlus | UltraPlus | UltraPlus | MaxiPlus | FAIL |
| 2 mismatches | MaxiPlus | UltraPlus | MaxiPlus | UltraPlus | FAIL |
| 3 mismatches | Deluxe | UltraPlus | MaxiPlus | MaxiPlus | FAIL |
| 4 mismatches | Deluxe | UltraPlus | MaxiPlus | Classic | FAIL |

Each FAIL result must individually name every mismatched source — e.g., a 2-mismatch case is reported as both “Bill OCR Mismatch” and “Corner Label Mismatch”, not as a single generic error.

## 4.5 Dimension Tolerance Check

Independently of product-identity matching, the Top Camera compares measured Length/Width/(Height) against the expected dimensions for the identified product and produces its own Pass/Fail dimension-verification status. A dimension failure should be reported alongside, but distinctly from, identity mismatches.

# 5. Data Model — Unified Inspection Database

All inspection results are stored in a JSON-based database. The schema below merges the fields defined separately for the Side and Top camera systems into one inspection record, since in practice both subsystems write to the same inspection ID.

## 5.1 Inspection Information

| Field | Type | Description |
| --- | --- | --- |
| inspection_id | string (unique) | Unique identifier for the inspection, shared across Side and Top records |
| timestamp | ISO 8601 datetime | Auto-generated at the start of processing |
| processing_status | enum: pass / fail | Final combined processing status |
| overall_verification_result | enum: PASS / FAIL | Final result after all four sources are compared |

## 5.2 QR Code Data (Side Camera)

| Field | Type | Description |
| --- | --- | --- |
| item_id | string | Item ID decoded from QR |
| batch_number | string | Batch number decoded from QR |
| product_variety | string | Product variety decoded from QR (e.g., MaxiPlus) |
| qr_code_value | string | Raw decoded QR payload |
| qr_metadata | object (optional) | Any additional encoded metadata |

## 5.3 OCR Results — Bill/Label (Side Camera)

| Field | Type | Description |
| --- | --- | --- |
| ocr_product_name | string | Product name/variety extracted from the bill/label |
| ocr_extracted_text | string | Full raw OCR text |
| ocr_confidence_score | float (0–1) | Model confidence for the OCR read |

## 5.4 Texture Detection Results (Side Camera)

| Field | Type | Description |
| --- | --- | --- |
| texture_predicted_category | string | Product category predicted by the texture model |
| texture_prediction_confidence | float (0–1) | Model confidence for the prediction |
| texture_model_version | string | Version identifier of the texture model used |

## 5.5 Dimension Results (Top Camera)

| Field | Type | Description |
| --- | --- | --- |
| measured_length | float | Measured length |
| measured_width | float | Measured width |
| measured_thickness | float (optional) | Measured height/thickness, if supported |
| expected_dimensions | object | Reference dimensions for the identified product |
| dimension_verification_status | enum: pass / fail | Result of comparing measured vs. expected, within tolerance |

## 5.6 Corner Label Results (Top Camera)

| Field | Type | Description |
| --- | --- | --- |
| label_product_name | string | Product name extracted from the corner label |
| label_product_variety | string | Product variety extracted from the corner label |
| label_product_code | string (optional) | Product code, if printed on the label |
| label_ocr_text | string | Full OCR text from the label |
| label_ocr_confidence | float (0–1) | Model confidence for the label OCR read |

## 5.7 Comparison Results

| Field | Type | Description |
| --- | --- | --- |
| qr_result | string | Product value from QR source |
| ocr_result | string | Product value from Bill OCR source |
| texture_result | string | Product value from Texture source |
| corner_label_result | string | Product value from Corner Label source |
| matching_status | enum: PASS / FAIL | Whether all four sources agree |
| mismatches | array of strings | List of mismatched source names, if any |
| remarks | string (optional) | Free-text notes, e.g. for manual review |

## 5.8 Image Storage

Images are never stored inline in the JSON record — only file paths are stored, keeping the database lightweight and portable.

| Field | Type | Description |
| --- | --- | --- |
| qr_image_path | string (path) | Path to the QR capture image |
| ocr_image_path | string (path) | Path to the Camera 2 image used for OCR |
| texture_image_path | string (path) | Path to the Camera 2 image used for texture detection |
| top_image_path | string (path) | Path to the full top-view image |
| corner_label_image_path | string (path) | Path to the cropped corner-label image |
| processed_image_path | string (optional) | Path to any annotated/processed image, if generated |

## 5.9 Sample Consolidated Record

{

        "inspection_id": "INSP-20260803-0042",

        "timestamp": "2026-08-03T09:14:22Z",

        "processing_status": "pass",

        "overall_verification_result": "PASS",

        "qr_data": {

          "item_id": "ITM-10245",

          "batch_number": "B-7781",

          "product_variety": "MaxiPlus",

          "qr_code_value": "ITM10245|B7781|MaxiPlus",

          "qr_metadata": {}

        },

        "ocr_results": {

          "ocr_product_name": "MaxiPlus",

          "ocr_extracted_text": "MaxiPlus - Premium Comfort",

          "ocr_confidence_score": 0.97

        },

        "texture_results": {

          "texture_predicted_category": "MaxiPlus",

          "texture_prediction_confidence": 0.94,

          "texture_model_version": "texture-v2.3"

        },

        "dimension_results": {

          "measured_length": 191.5,

          "measured_width": 91.2,

          "measured_thickness": 25.0,

          "expected_dimensions": { "length": 190, "width": 90, "thickness": 25 },

          "dimension_verification_status": "pass"

        },

        "corner_label_results": {

          "label_product_name": "MaxiPlus",

          "label_product_variety": "MaxiPlus",

          "label_product_code": "MP-2026",

          "label_ocr_text": "MaxiPlus MP-2026 King Size",

          "label_ocr_confidence": 0.96

        },

        "comparison_results": {

          "qr_result": "MaxiPlus",

          "ocr_result": "MaxiPlus",

          "texture_result": "MaxiPlus",

          "corner_label_result": "MaxiPlus",

          "matching_status": "PASS",

          "mismatches": [],

          "remarks": ""

        },

        "image_paths": {

          "qr_image_path": "/images/2026-08-03/INSP-0042/qr.jpg",

          "ocr_image_path": "/images/2026-08-03/INSP-0042/ocr.jpg",

          "texture_image_path": "/images/2026-08-03/INSP-0042/texture.jpg",

          "top_image_path": "/images/2026-08-03/INSP-0042/top.jpg",

          "corner_label_image_path": "/images/2026-08-03/INSP-0042/label.jpg",

          "processed_image_path": null

        }

      }

# 6. Dashboard Requirements

A single dashboard serves both subsystems so an operator can run a full inspection without switching screens.

## 6.1 Live Camera Views

- Live feed from Camera 1 (QR Scanner)

- Live feed from Camera 2 (OCR & Texture)

- Live feed from the Top Camera (Dimension & Corner Label)

## 6.2 Process Button

When the operator clicks Process, the system should perform the full combined sequence:

- 1. Capture images from all three cameras.

- 2. Decode the QR code.

- 3. Run Bill/Label OCR.

- 4. Run Texture Detection.

- 5. Measure mattress dimensions.

- 6. Detect and OCR the corner label.

- 7. Compare all four identification sources.

- 8. Compare measured vs. expected dimensions.

- 9. Generate the final verification result.

- 10. Display the result on the dashboard.

- 11. Save the complete inspection record, including all image paths, to the database.

## 6.3 Result Panel

- PASS / FAIL status

- QR Code, Bill OCR, Texture, and Corner Label results

- Product Variety, Item ID, Batch Number

- Measured vs. expected dimensions and dimension pass/fail

- Timestamp

- Confidence scores for OCR, texture, and label reads

- List of mismatches, if any

## 6.4 Inspection History & Search

Searchable by:

- Inspection ID

- Item ID

- Product Variety

- Batch Number

- Date / Timestamp

- Inspection Status (Pass/Fail)

Selecting a record displays all inspection details, every captured image, verification and dimension results, mismatch details, and confidence scores.

# 7. Cross-Cutting Features

Consolidated from both source specifications; duplicated items are merged.

- Automatic timestamp generation for every inspection

- Unique inspection ID for traceability, shared across Side and Top records

- Image archiving with an organized folder structure

- JSON-based inspection logs

- Confidence scores for OCR, texture, and corner-label predictions

- Dimension tolerance checking

- Error handling for unreadable QR codes, OCR failures, or unreadable labels

- Camera connection status monitoring for all three cameras

- Processing progress indicator

- Manual reprocessing option for failed inspections

- Export inspection history (JSON/CSV) for reporting

- Audit logs for all inspection activity

- Scalable database structure to support future product categories and additional verification sources

# 8. End-to-End Workflow

- Operator places the product in the inspection area, under both the Side and Top cameras.

- Camera 1 scans and decodes the QR code.

- Camera 2 captures an image used for both OCR and texture detection.

- The QR Processing module extracts Item ID, Batch Number, and Product Variety.

- The OCR & Texture Processing module reads the bill/label and classifies the product texture.

- The Top Camera captures a top-view image.

- The Dimension Measurement module measures Length, Width, and (optionally) Thickness.

- The Corner Label Reading module detects, crops, and OCRs the corner label.

- The system compares QR, Bill OCR, Texture, and Corner Label results against one another.

- The system separately compares measured dimensions against the expected dimensions for the identified product.

- If all four identification sources agree (and dimensions are within tolerance), the inspection is marked PASS.

- If one or more sources disagree, the inspection is marked FAIL, and every mismatched source is individually listed.

- The complete inspection record — identification results, dimension results, comparison results, image paths, and timestamp — is saved to the JSON database under a single inspection ID.

- The dashboard updates with the latest result and makes it available for future search and review.

# 9. Assumptions & Dependencies

- The OCR and texture-detection AI models are pre-trained and delivered ready for integration; no model training is part of this scope.

- The Side Camera System and Top Camera System write to the same inspection database and share a single inspection_id per product pass.

- “Expected dimensions” and reference product categories are sourced from a product master table that is assumed to exist but is not defined in either source specification.

- Camera 1, Camera 2, and the Top Camera can be triggered either simultaneously or in a fixed sequence from a single Process action; the exact orchestration is an implementation decision (see Section 10).

- Network/storage infrastructure for image archiving is available and sized for continuous production-line throughput.

- “Batch Number” and “Item ID” are only produced by the QR source; the Top Camera's corner label is assumed to identify product name/variety/code only, not batch or item-level data.

# 10. Open Questions for Engineering

These items were not fully resolved by combining the two original specifications and should be confirmed with product/engineering stakeholders before implementation.

- Is there a single Process button that triggers all three cameras together, or separate Process actions for the Side and Top systems that are later reconciled into one record?

- Should the Stage 1 (3-source) result be shown to the operator as an interim status while waiting for the Top Camera, or only logged internally?

- What is the source of truth for “expected dimensions” per product variety, and how is it kept in sync with new product categories?

- What is the required behavior when the QR code is unreadable but OCR, Texture, and Corner Label all agree — is a manual override permitted, and if so, who can authorize it?

- What confidence-score threshold, if any, should cause an automatic FAIL even when the predicted category matches (e.g., a low-confidence texture prediction that happens to match)?

- How should the system handle two inspections in progress at the same time (e.g., a fast-moving line) without cross-contaminating captured images or inspection IDs?

- What is the retention policy for archived images and JSON records, and is there a maximum database/storage size to plan for?

- Does “manual reprocessing” re-run all four sources, or allow an operator to re-run a single failed source (e.g., only re-scan the QR code)?

# 11. Glossary

| Term | Definition |
| --- | --- |
| QR Processing | The Side Camera module that captures and decodes the product QR code. |
| OCR & Texture Processing | The Side Camera module that reads printed text (bill/label) and classifies surface texture from the same captured image. |
| Corner Label Reading | The Top Camera module that detects, crops, and OCRs the label printed on the mattress corner. |
| Dimension Measurement | The Top Camera module that measures the physical Length, Width, and optionally Thickness of the mattress. |
| Matching Status | The overall PASS/FAIL result of comparing all four identification sources. |
| Mismatch | A case where one verification source reports a product category different from the others; each mismatch is individually named in the result. |
| Inspection Record | The single JSON document capturing every field, image path, and result for one product pass through the system. |
| Product Variety | The specific product line/category being verified, e.g., MaxiPlus, UltraPlus, Deluxe, Classic. |